# Benton County Tax Number

# Benton County Tax/Parcel Number format is a 15 digit number. Digits 2 thru 5 describes the township/range/section of the parcel. Digit 6 describes the location of the parcel within the quarter section. Digits 7 and 8 designates what type of parcel it is. Digits 9 thru 12 describes the blocks and Tracts. Digits describe the lot numbers.

# Plats

Parcel #: 1-2345-6XX-9012-345, the first character will be assigned based on the guidelines below.

1 = Real Parcel

4 = Mobile Home

7 = Railroad Lease

8 = Senior Citizen Lease

9 = Leased Residential Improvement (example: BSP 5828)

B = Building

Digits 2 and 3 are the Map Section: As shown on Plat (01 thru 36)

- If there are multiple sections, the new child parcel will follow its section location. (example: SP 3781)

Digit 4 the Township: As shown on Plat (4 thru 14)

Digit 5 the Range: As shown on Plat (24 thru 31)

Digit 6 describes the location of the parcel (1-2289-2)

- Quarter Section Number:

1 = North East Quarter Section

2 = North West Quarter Section

3 = South West Quarter Section

4 = South East Quarter Section

- Plat – The quarter section is determined by the plat map and location of the new parcel.

  - If there are multiple quarter sections, the new child parcel will follow its quarter section location.

Digits 7 and 8 designations the parcel type.

- Plats – are assigned (1-2289-202 thru 1-2389-299) in sequential order of the last subdivision number in that quarter section.

  - If the last subdivision was (1-2289-203) the next subdivision will be given (1-2289-204)

  - Altered/Amended Plat – digits 7 and 8 will be assigned a new number after the last subdivision in the quarter section

Digits 9 thru 12 Block and Tracts

- Plat – block numbers are assigned (1-2289-204-0001 thru 1-2289-204-0099) tracts are numbered sequentially after the last block number.

- Block 1 thru block 4 will be  1-2289-204-0001 thru 1-2289-204-0004

- Tracts A thru B will be 1-2289-204-0005 thru 1-2289-204-0006

Digits 13 thru 15 Lots

- Plat – lot numbers (1-2289-204-0001-001) and tracts have a (1-2289-204-0001-000) for a lot number

South Hill Phase 1 parcels

Block 1                                1-2289-204-0001-001

Block 2                                1-2289-204-0002-001 thru 1-2289-204-0002-009

Block 3                                1-2289-204-0003-001 thru 1-2289-204-0003-003

Block 4                                1-2289-204-0004-001 thru 1-2289-204-0004-008

Tracts A thru B                  1-2289-204-0005-000 thru 1-2289-204-0006-000

# Short Plats

Parcel #: 1-2345-6XX-9012-345, the first character will be assigned based on the guidelines below.

1 = Real Parcel

4 = Mobile Home

7 = Railroad Lease

8 = Senior Citizen Lease

9 = Leased Residential Improvement (example: BSP 5828)

B = Building

Digits 2 and 3 are the Map Section: As shown on Plat (01 thru 36)

- If there are multiple sections, the new child parcel will follow its section location. (example: SP 3781)

Digit 4 the Township: As shown on Plat (4 thru 14)

Digit 5 the Range: As shown on Plat (24 thru 31)

Digit 6 describes the location of the parcel (1-3299-1)

- Quarter Section Number:

1 = North East Quarter Section

2 = North West Quarter Section

3 = South West Quarter Section

4 = South East Quarter Section

- Short Plat – Quarter Section is determined by the location of the short plat

  - The parent parcel is 1-3299-401-2374-004  (Quarter Section 32994)

  - Short plat boundary is located in Quarter Section 32991

Digits 7 and 8 designations the parcel type.

- Short Plat – are assigned (1-3399-101)

Digits 9 thru 12 Block

- Short Plat – will be the recorded Short Plat Number (1-3399-101-3337)

  - Altered/Amended Short Plat – original parcels will be retired and new parcel numbers will be assigned to the altered/amended short plat

Digits 13 thru 15 Lots and Tracts

- Short Plat – lot number (1-3399-101-3337-001)

- Tracts on the short plat will be given the next number after the lot number

Short Plat 3337

1-3399-101-3337-001

1-3399-101-3337-002

# Binding Site Plans

Parcel #: 1-2345-6XX-9012-345, the first character will be assigned based on the guidelines below.

1 = Real Parcel

4 = Mobile Home

7 = Railroad Lease

8 = Senior Citizen Lease

9 = Leased Residential Improvement (example: BSP 5828)

B = Building

Digits 2 and 3 are the Map Section: As shown on Plat (01 thru 36)

- If there are multiple sections, the new child parcel will follow its section location. (example: SP 3781)

Digit 4 the Township: As shown on Plat (8 or 9)

Digit 5 the Range: As shown on Plat (28, 29 or 30)

Digit 6 describes the location of the parcel (1-0689-3)

- Quarter Section Number:

1 = North East Quarter Section

2 = North West Quarter Section

3 = South West Quarter Section

4 = South East Quarter Section

- Binding Site Plan – Quarter section is determined by the location of the binding site plan.

  - The parent parcels are 1-0188-4BP-4645-003 and 1-0689-300-0002-011:

    - If there are multiple quarter sections, the new child parcel will follow its quarter section location.

Digits 7 and 8 designations the parcel type.

- Binding Site Plan – are assigned (1-0689-3BP)

Digits 9 thru 12 Block and Tracts

- Binding Site Plan – will be the recorded Binding Site Plan Number (1-0689-3BP-4740)

  - Altered/Amended Binding Site Plan – original parcels will be retired and new parcel numbers will be assigned to the altered/amended Binding Site Plan

Digits 13 thru 15 Lots

- Binding Site Plan – lot number (1-0689-3BP-4740-001)

Binding Site Plan

1-0689-3BP-4740-001 thru 1-0689-3BP-4740-007

# Record Surveys/Metes & Bounds

# Benton County Tax/Parcel Number format is a 15 digit number. Digits 2 thru 5 describes the township/range/section of the parcel. Digit 6 describes the location of the parcel within the quarter section. Digits 7 and 8 designates what type of parcel it is. Digits 9 thru 12 describes the blocks and Tracts. Digits describe the lot numbers.

Parcel #: 1-2345-6XX-9012-345, the first character will be assigned based on the guidelines below.

1 = Real Parcel

4 = Mobile Home

7 = Railroad Lease

8 = Senior Citizen Lease

9 = Leased Residential Improvement (example: BSP 5828)

B = Building

Digits 2 and 3 are the Map Section: As shown on Plat (01 thru 36)

Digit 4 the Township: As shown on Plat (8 or 9)

Digit 5 the Range: As shown on Plat (28, 29 or 30)

Digit 6 describes the location of the parcel (1-2389-1)

- Quarter Section Number:

1 = North East Quarter Section

2 = North West Quarter Section

3 = South West Quarter Section

4 = South East Quarter Section

- Record Survey/Non

Digits 7 and 8 designations the parcel type.

- Record Survey/Non-platted – are assigned (1-2389-100)

- Metes and Bounds – digits 7 and 8 should match the parent parcel

Digits 9 thru 12 Block and Tracts

- Metes and Bounds – will have the same number as the parent parcel (1-2389-102-0002 thru 1-2389-102-0099)

Digits 13 thru 15 Lots

- Metes and Bounds – lot number will be assigned the next number in the original parent parcel number